- Bacteriological techniques

There are several techniques used in a microbiology laboratory, they can be classified into the following processes.

- Inspection- Physical observation of the morphological characteristics of the bacteria via a microscope.

Microscopes magnify images and use the properties of light to create useful images of small objects. Three properties are exploited in microscopes to enable viewing of microorganisms;

-Magnification which is defined as the ability of a lens to enlarge the I mage of an object when compared to the real object. For example, a magnification of 10⨯ means that the image appears 10 times the size of the object as viewed with the naked eye.

-Magnification is used together with resolution. Resolution is the ability to differentiate two separate points or two separate objects.

- Low resolution images appear fuzzy, whereas a high-resolution image appears sharp.

-Even when a microscope has high resolution, it can be difficult to distinguish small structures in many specimens because microorganisms are relatively transparent. It is often necessary to increase contrast to detect different structures in a specimen.

- Different types of microscopes use different features of light or electrons to increase contrast.

- Dyes are also used to stain some structures of the microorganisms for better contrast.

Types of Microscopes

Light microscope

The light microscope uses light or visible radiation to visualize images. Examples of light microscopes include;

- brightfield microscopes which has two or more lenses that produce a dark image on a bright background.

- darkfield microscopes which will show a bright image on a dark background

- fluorescence microscopes (use of fluorochromes which are capable of absorbing energy from light source and emitting the light)

- phase contrast microscopes (creates image by altering the wavelengths of light rays passing through the specimen) etc.

Most light microscopes can only magnify 1000x and a few up to 1500x

Components of a typical light microscope. Images seen under a brightfield (a) and darkfield light (b) microscope (https://bio.libretexts.org/Courses/Mansfield_University_of_Pennsylvania/BSC_3271%3A_Microbiology_for_Health_Sciences_Sp21_(Kagle)/01%3A_Introduction/1.03%3A_How_We_See_the_Invisible_World/1.3.03%3A_Instruments_of_Microscopy)

Electron Microscope (EM)

Electron microscopes use short-wavelength electron beams rather than light to increase magnification and resolution of images.

An EM can produce a sharp image that is magnified up to 100,000x and so can resolve subcellular structures as well as some molecular structures like single strands of DNA.

Electron microscopes cannot be used to view living material because of the methods used to prepare the samples.

There are two types of EM, transmission electron microscope (TEM) and scanning electron microscope (SEM). TEM requires that the specimen should be very thin and allows view of the organelles while SEM allows viewing of the surfaces of the organism.

(a) TEM image of cells in a biofilm shows a well-defined internal structures of the cells because of varying levels of opacity in the specimen. (b) A color-enhanced SEM image of the bacterium Staphylococcus aureus illustrates the ability of scanning electron microscopy to render three-dimensional images of the surface structure of cells.

A scanning probe microscope does not use light or electrons, but very sharp probes are passed over the surface of the specimen and interact with it. Images produced can be magnified up to 100,000,000x. This type of microscopy is mostly used for research rather than diagnostics to observe individual atoms on surfaces.

- Identification- classifying microorganisms via color, shape, size, smell.

Being able to identify bacteria under the microscope requires staining. Bacterial staining is the process of coloring of colorless bacterial structural components using stains (dyes).

The principle of staining is to identify microorganisms selectively by using dyes, fluorescence, and radioisotope emission.

Staining reactions are made possible because of the physical phenomena of capillary osmosis, solubility, adsorption and absorption of stains or dyes by cells of microorganisms.

Individual variation in the cell wall constituents among different groups of bacteria will consequently produce variations in colors during microscopic examination.

Different types of affinity bonds are responsible for the attraction between dyes and bacterial structures: hydrophobic bonding, reagent-cell interaction, reagent-reagent interaction, ionic bonding, hydrogen bonding and covalent bonding.

Types of microbiological stains

Basic stains like carbolfuchsin, methylene blue, crystal violet. Acidic stains e.g., eosin stain, acid fuchsin, rose bengal etc.

Neutral dyes stain both nucleic acid and cytoplasm e.g., Giemsa stain

Staining Methods

1. Simple staining method – this staining method makes use of only a single dye usually to demonstrate bacterial morphology and arrangement.

Simple staining could either be positive (where the bacteria and the parts are stained) e.g., carbol-fuschin stain and crystal violet stain or negative where the background is stained, but the bacteria remain unstained e.g., Indian ink stain.

Procedure of simple staining

-Make a smear and label it

-Allow the smear to dry in air

-Fix the smear over a flame

-Apply a few drops of simp  le stain like 1% methylene blue, 1% carbolfuchsin or 1% gentian violet for 1 minute.

-Wash off the stain with water

-Air-dry and examine under the oil immersion objective

2. Differential staining method - Multiple stains are used in differential staining method to distinguish different cell structures and/or cell types. E.g. Gram stain and Ziehl-Neelson stain.

Gram staining differentiates bacteria based on differences in their cell wall structure. Gram-positive bacteria will stain purple while gram-negative bacteria stain pink.

Procedure for Gram stain

-Prepare the smear from the culture or from the specimen.

-Allow the smear to air-dry completely.

-Rapidly pass the slide (smear upper most) three times through the flame.

-Cover the fixed smear with crystal violet for 1 minute and wash with distilled water.

-Tip off the water and cover the smear with gram’s iodine for 1 minute.

-Wash off the iodine with clean water.

-Decolorize rapidly with acetone-alcohol for 30 seconds.

-Wash off the acetone-alcohol with clean water.

-Cover the smear with safranin for 1 minute.

-Wash off the stain wipe the back of the slide and let the smear air-dry.

-Examine the smear with oil immersion objective to look for bacteria.

Ziehl-Neelson staining method also called Acid-fast staining is used for staining Mycobacteria which cannot be stained by the gram staining method. Once the Mycobacteria is stained with primary stain it cannot be decolorized with acid, so named as acid-fast bacteria.

Procedure for Ziehl-Neelson staining method

-Prepare the smear from the specimen and fix it by passing through the flame and label clearly

-Place fixed slide on a staining rack and cover each slide with concentrated carbol-fuchsin solution.

-Heat the slide from underneath with sprit lamp until vapor rises (do not boil it) and wait for 3-5 minutes.

-Wash off the stain with clean water.

-Cover the smear with 3% acid-alcohol solution until all color is removed (two minutes)

-Wash off the stain and cover the slide with 1% methylene blue for one minute

-Wash off the stain with clean water and let it air-dry and examine the smear under the oil immersion objective to look for acid fast bailli.

Interpretation: Acid fast bacilli…………...Red on a blue background

3. Special staining method

Special stains

- Spore staining method

-Prepare smear of the spore-forming bacteria and fix in flame.

-Cover the smear with 5% malachite green solution and heat over steaming water bath for 2-3 minutes.

-Wash with clean water.

-Apply 1% safranin for 30 seconds.

-Wash with clean water.

-Dry and examine under the oil immersion objective.

- Capsule staining method- Welch method Procedure:

-Prepare smear of capsulated bacteria.

-Allow smear to air-dry; do not fix the smear.

-Cover the smear with 1% aqueous crystal violet for 1 minute over steaming water bath. --Wash with 20% copper sulfate solution. Do not use water.

-Dry and examine under the oil immersion objective.

- Inoculation- involves introduction of microorganisms such as bacteria into culture media.

Culture media contains all the nutrients required by the bacteria for growth and development. Common ingredients of culture media are:

- Peptone (a hydrolysed form of plant and animal proteins which supply amino acids, Nitrogen)

- Meat extract (supply amino acids, vitamins and minerals),

- Yeast extract (stimulates bacterial growth)

- Mineral salts (like sulphur from sulphates, phosphorus from phosphates etc)

- Carbohydrates (simple and complex carbohydrates),

- Agar (growth support) and water (deionized or distilled).

Types of media/agar

1.Basic /Simple / All-purpose media- It is media that supports the growth of micro-organisms that do not require special nutrients.

2. Enriched media are enriched with whole blood, lyzed blood, serum, special extracts, or vitamins to support the growth of pathogenic bacteria e.g. Blood Agar, Chocolate Agar

3. Enrichment media is fluid media containing enrichments and/or substances that discourage the multiplication of unwanted bacteria. e.g. Selenite F broth media, Alkaline peptone water

4. Selective media contain substances like antibiotics that prevent or slow down the growth of bacteria other than pathogens for which the media are intended. Eg. Modified Thayer –Martin Agar, Salmonella-Shigella (SS) agar.

5.Differential media is media to which indicator substances are added to differentiate bacteria. e.g., TCBS Agar differentiates sucrose fermenting yellow colonies of Vibrio cholerae to non-sucrose fermenting blue colonies other Vibrio species.

NB: Most differential media distinguish between bacteria by an indicator which changes color when acid is produced following carbohydrate fermentation.

6. Transport media contains ingredients to prevent the overgrowth of commensals and ensure the survival of pathogenic bacteria when specimens cannot be cultured soon after collection.

There are 3 main methods of inoculating and culturing bacteria;

- Streak plate method: In this technique, an inoculation loop is dipped in a diluted microorganism suspension and streaked on the solid surface of the culture medium. The plate which gets streaked is known as the streak plate.

The method is generally used to obtain individual bacteria colonies from a concentrated suspension or to prepare a pure culture of a bacterial strain.

Demonstration of streak plate method of inoculation (https://conductscience.com/microbiology-techniques/)

- Spread plate method: In this method, a very small volume of the liquid suspension of the microorganism is poured on the solidified surface of the media-containing plate. Then, an L-shaped glass rod is used to spread the liquid evenly all over the plate surface.

This is performed to obtain individual colonies of the microorganisms but can also count the number of the microbial population.

- Pour plate method: In this technique, a serially diluted microorganism suspension is pipetted in a sterile petri dish. Then, a liquified, cooled culture media is poured into the plate. After the media is solidified, the culture plate is incubated for specific bacterial growth.

It’s performed to estimate the viable bacterial count in a microbial suspension.

Illustration of Pour plate technique of bacterial inoculation ((https://conductscience.com/microbiology-techniques/)

- Incubation- for days or weeks for the microorganism to grow and multiply.

Inoculated media should be incubated as soon as possible. Optimal temperature, humidity and gaseous atmosphere should be provided for microorganisms to grow best.

The temperature selected for routine culturing is 35-37oC. Some pathogens require CO2-enriched atmosphere to grow in culture media, and the simplest way to provide CO2-enriched atmosphere is to enclose a lighted candle in an airtight jar which provides 3-5% CO2 by the time the candle is extinguished.

Anaerobic atmosphere is essential for the growth of strict anaerobes, and the techniques for obtaining anaerobic conditions are the following: -Anaerobic jar with a gas generating kit. Reducing agents in culture media.

- Isolation – various colonies and how they look like. How to separate them from the rest.

The colonies on the plate could be counted, number of bacteria can be counted under the microscope.

Since colonies usually contain high concentrations of bacteria which makes it impossible to count, serial dilutions are used to reduce the number.

Also, some biochemical tests are performed on cultured bacteria to identify the various types of bacteria. Common ones include:

- CATALASE TEST

This test is used to differentiate those bacteria that produce the enzyme catalase, such as staphylococci, from non-catalase producing bacteria such as streptococci.

Principle

Catalase acts as a catalyst in the breakdown of hydrogen peroxide to oxygen and water.

An organism is tested for catalase production by bringing it into contact with hydrogen peroxide. Bubbles of oxygen are released if the organism is a catalase producer.

The culture should not be more than 24 hours old. Care must be taken when testing an organism cultured on a medium containing blood because catalase is present in red cells.

It is usually recommended, therefore, that catalase testing be performed on a blood free culture medium such as nutrient agar.

Method

• Pour 2-3ml of the hydrogen peroxide solution into a test tube

• Using a sterile wooden stick or a glass rod, remove a good growth of the test organism and immerse it in the hydrogen peroxide solution.

Note: A nichrome wire loop must not be used because this may give a false positive reaction.

• Look for immediate bubbling.

Active bubbling ----------------- Positive test-Catalase produced

No release of bubbles ---------- Negative test-No catalase produced

- CITRATE UTILIZATION TEST This test is one of several techniques used to assist in the identification of enterobacteria.

The test is based on the ability of an organism to use citrate as its only source of carbon and ammonia as its only source of nitrogen.

Principle

The test organism is cultured in a medium which contains sodium citrate, an ammonium salt, and the indicator, bromo-thymol blue. Growth in the medium is shown by turbidity and a change in color of the indicator from light green to blue, due to the alkaline reaction, following citrate utilization.

Method

-Using a sterile straight wire, inoculate 3-4ml of sterile Koser’s citrate medium with a broth culture of the test organism.

Note: Care must be taken not to contaminate the medium with carbon particles, such as from a frequently flamed wire.

-Incubate the inoculated broth at 35 – 37OC for up to 4 days, checking daily for growth. Results

Turbidity and blue colour ------------------------------------ Positive test -Citrate utilized  (e.g. Klebsiella pneumoniae)

No growth ----------------------------------------------Negative test -Citrate not utilized (e.g. E. coli)

- COAGULASE TEST

This test is used to differentiate Staphylococcus aureus which produces the enzyme coagulase, from S.epidermidis and S.saprophyticus which do not produce coagulase.

Principle

Coagulase causes plasma to clot by converting fibrinogen to fibrin.

Method

- Place a drop of physiological saline on each end of a slide, or on two separate slides.

- Add a colony of the test organism in each of the drops to make two thick suspensions.

Note: Colonies from a mannitol salt agar culture are not suitable for coagulase testing. The organism must first be cultured on nutrient agar or blood agar.

-Add a drop of plasma to one of the suspensions and mix gently. Look for clumping of the organisms within 10 seconds. No plasma is added to the second suspension. This is used to differentiate any granular appearance of the organism form true coagulase clumping.

Clamping within 10 secs ------------------------ S.aureus

No clumping within 10 secs-----------No bound coagulase produced.

- DEOXYRIBONUCLEASE (DNAse) TEST This test is used to differentiate S. aureus which produces the enzyme DNAse from other staphylococci which do not produce DNAse.

It is particularly useful if plasma is not available to perform a coagulase test or when the results of a coagulase test are difficult to interpret.

Principle

Deoxyribonuclease hydrolyzes deoxyribonucleic acid (DNA). The test organism is cultured on a medium which contains DNA. After overnight incubation, the colonies are tested for DNAse production by flooding the plate with a weak hydrochloric acid solution. The acid precipitates unhydrolyzed DNA. DNAse producing colonies are, therefore surrounded by clear areas indicating DNA hydrolysis.

Method

-Divide a DNAse plate into the required number of strips by marking the underside of the plate

-Using a sterile loop or swab to inoculate the test and control organisms.

-Make sure each test area is clearly labeled.

-Incubate the plate at 36-37oC overnight.

-Cover the surface of the plate with 1mol/l hydrochloric acid solution. Tip off the excess acid.

-Look for clearing around the colonies within 5minutes of adding the acid.

Results

Clearing around the colonies -----------------DNAse positive strain (Staph aureus).

No clearing around to colonies -------------DNAse negative strain (Staph epiderdimis).

- OXIDASE TEST (Cytochrome Oxidase) The oxidase test is used to assist in the identification of pseudomonas, Neisseria, Vibrio, and Pasteurella species, all of which produce oxidase enzymes.

Principle

A piece of filter paper is soaked with a few drops of oxidase reagent. A colony of the test organism is then smeared on the filter paper. If the organism is oxidase - producing, the phenylenediamine in the reagent will be oxidized to a deep purple colour. Occasionally the test is performed by flooding the culture plate with oxidase reagent but this technique is not recommended for routine use because the reagent rapidly kills bacteria.

Method

-Place a piece of filter paper in a clean petri dish and add 2 or 3 drops of freshly prepared oxidase reagent.

-Using a piece of stick or glass rod (not an oxidized wire loop), remove a colony of the test organism, and smear it on the filter paper.

-Look for the development of a blue – purple colour within a few seconds

Results

Blue-purple colour ------------------------------------- Positive test (within 10 seconds) Oxidase produced e.g., Pseudomonas aeruginosa

No blue – purple colour ------------------------------- Negative test (within 10 seconds) No oxidase produced e.g., Escherichia coli.

Methods of sterilization and preparation of media cultures

Most culture media are available commercially in ready-made dehydrated form. However, the major processes during preparation of culture media are;

- Weighing and dissolving of culture media ingredients:

• Use ingredients suitable for microbiological use.

• Use clean glass ware, plastic or stainless-steel equipment.

• Use distilled water.

• Do not open new containers of media before finishing previous ones.

• Weigh in a cool, clean, dry and draught-free atmosphere.

• Weigh accurately using a balance.

• Wear a facemask and glove while weighing and dissolving toxic chemicals.

• Do not delay in making up the medium after weighing. Add powdered ingredients to distilled water and mix by rotating or stirring the flask.

• Stir while heating if heating is required to dissolve the medium.

• Autoclave the medium when the ingredients are dissolved.

- Sterilization and sterility testing:

-Always sterilize a medium at the correct temperature and for the correct length of time as instructed in the method of preparation.

Methods used to sterilize culture media;

- Autoclaving which is used to sterilize most agar and fluid culture media.

- Steaming at 1000C-It is used to sterilize media containing ingredients that would be inactivated at temperature over 1000C and re-melt previously bottled sterile agar media.

- Filtration-It is used to sterilize additives that are heat-sensitive and cannot be autoclaved.

Sterility testing is the simplest way to test for contamination by incubating the prepared sample media at 35-370C for 24 hours.

Turbidity in fluid media and microbial growth in solid media confirm contamination.

- Addition of heat-sensitive ingredients: Refrigerated-heat sensitive ingredients should be warmed at room temperature before added to a molten agar medium. Using an aseptic technique, the ingredients should be added when the medium has cooled to 50oC and should be distributed immediately unless further heating is required.

- pH testing: The pH of most culture media is near neutral and can be tested using a pH meter.

- Dispensing of culture media: Media should be dispensed in a clean draught-free room using aseptic technique and sterile container.

To dispense media into a petri dish, lay out the sterile Petri dishes on a level surface, mix the medium gently by rotating the flask or bottle, flame sterilize the neck of flask or bottle, pour 15 ml of medium in each Petri dish.

• Stack the plates after the medium has gelled or cooled and store the plates in a refrigerator.

NB: Agar plates should be of an even depth and of a firm gel. The surface of the medium should be smooth and free from bubbles.

- Quality control is required to make the quality of media is good

• Inoculate quarter plates of the medium with a five hours broth culture for each control organism.

• Use a straight wire to inoculate and wire loop to spread the inoculum.

• Depending on the species, incubate aerobically, CO2- enriched atmosphere and anaerobically at 35-37oC for 24 hours.

• Examine for the degree of growth, morphology and other characteristics of microbial colonies.

• Record the result of each control species and compare to your standard reading.

Storage of culture media

• Dehydrated culture media and dry ingredients should be stored at an even temperature in a cool dry place away from direct light.

• Plates of culture media, and additives like serum, blood and antimicrobials in solid form require storage at 2-8oC.

• Antimicrobials in solution form should be stored at –20oC.

• All culture media and additives should be labeled with the name and date of preparation.

Note*-: Sterilization is the destruction of all forms of microbial life including spores.

Disinfection is the destruction of microbes that cause disease; may not be effective in killing spores.

Antisepsis is the destruction or inhibition of microorganisms in living tissue there by limiting or preventing the harmful effect of infection.